EX NO: 7

CALCULATION AND VERIFICATION OF FREQUENCY

DEVIATION AND MODULATION INDEX OF FM SIGNALS USING

MATLAB SIMULATION

Objectives:

1. To generate a Frequency Modulated (FM) signal in MATLAB using specified carrier and

message signal parameters.

2. To analyze the effect of message frequency on frequency deviation and modulation

index, and verify their relationship through MATLAB simulation

PROGRAM (MATLAB CODE)

clc;

clear;

close all;

%% Message Signal Parameters

Am = 2; % Message amplitude (V)

fm = 500; % Message frequency (Hz)

%% Carrier Signal Parameters

Ac = 1; % Carrier amplitude (V)

fc = 10000; % Carrier frequency (Hz)

%% FM Parameters

kf = 250; % Frequency sensitivity (Hz/V)

fs = 100000; % Sampling frequency (Hz)

T = 0.01; % Simulation duration (s)

%% Time Vector

t = 0:1/fs:T;

%% Message Signal

m = Am*cos(2*pi*fm*t);

%% Frequency Deviation

delta_f = kf*Am;

%% Modulation Index

beta = delta_f/fm;

%% FM Signal

fm_signal = Ac*cos(2*pi*fc*t + beta*sin(2*pi*fm*t));

%% Display Parameters

fprintf('Carrier Frequency (fc) = %.0f Hz\n',fc);

fprintf('Message Frequency (fm) = %.0f Hz\n',fm);

fprintf('Message Amplitude (Am) = %.0f V\n',Am);

fprintf('Frequency Sensitivity (kf) = %.0f Hz/V\n',kf);

fprintf('\nFrequency Deviation (Delta f) = %.2f Hz\n',delta_f);

fprintf('Modulation Index (Beta) = %.2f\n',beta);

%% Verification

beta_verify = delta_f/fm;

fprintf('Verified Modulation Index = %.2f\n',beta_verify);

%% Verification Check

tolerance = 1e-6;

if abs(beta - beta_verify) < tolerance

fprintf('\nVerification Successful\n');

else

fprintf('\nVerification Failed\n');

end

%% Plotting

figure;

subplot(2,1,1);

plot(t,m,'LineWidth',1.5);

grid on;

title('Message Signal');

xlabel('Time (s)');

ylabel('Amplitude (V)');

subplot(2,1,2);

plot(t,fm_signal,'LineWidth',1);

grid on;

title('FM Signal');

xlabel('Time (s)');

ylabel('Amplitude (V)');

Output Map: